| ### 4,000 lb. Maximum Lifting Capacity 16,000 ft.-lb. Maximum Moment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Compliance to Specifications | The Bidder shall indicate their compliance by checking "Yes" or "No" for each item specified. Any space not checked shall be interpreted as non-compliance and will result in disqualification of the bid. |  |  |  |  |
| Make & Model | ##### EHC-4 NexStar 16’ Boom |  | Comply to Specification |  | ## Deviation from Specification |
|  |  |  | Yes | No |  |
| #### Capacity (Measured from Center of Rotation) | 4,000 lb. @ 4 ft. |  |  |  |  |
|  | 2,666 lb. @ 6 ft. |  |  |  |  |
|  | 2,000 lb. @ 8 ft. |  |  |  |  |
| (See full load and stability charts attached for reference) | 1,600 lb. @ 10 ft. |  |  |  |  |
|  | 1,333 lb. @ 12 ft. |  |  |  |  |
|  | 1,143 lb. @ 14 ft. |  |  |  |  |
|  | 1,000 lb. @ 16 ft. |  |  |  |  |
| #### Specifications & Standards | Overall dimensions (L x W x H) 116 in. x 20.63 in. x 24 in. |  |  |  |  |
|  | Weight of crane shall not exceed 960 lb. |  |  |  |  |
|  | Minimum truck chassis of: - 10,500 lb. GVWR |  |  |  |  |
|  | - 210,600 in.-lb. RBM |  |  |  |  |
|  | Paint – Auto Crane White |  |  |  |  |
|  | Compliance with ANSI B30.5 standards and OSHA regulations concerning crawler locomotives and truck cranes (OSHA Title 29, Part 1910.180) |  |  |  |  |

| DC Power Unit | Electro-hydraulic system consists of an electric motor driving hydraulic pump which Provides fluid power to: - Boom Lift / Lower |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Boom Extension / Retraction |  |  |  |
|  | - Rotation |  |  |  |
|  | - Hoist Up / Down |  |  |  |
|  | Electric motor – 12V DC series wound |  |  |  |
|  | Hydraulic: - Gear pump, .065 cu. in./rev. displacement |  |  |  |
|  | - 2.4 gallon nominal-capacity reservoir |  |  |  |
|  | System: - Set full by-pass relief @ 3,000 PSI |  |  |  |
|  | - Output: 1.0 GPM minimum @ 2,750 PSI @ 239 amps max. |  |  |  |

| Boom – Lift | Hydraulically actuated |  |  |  |
| --- | --- | --- | --- | --- |
|  | Lift angle from -3° to 75° |  |  |  |
|  | Lift cylinder: - Double acting for smooth boom operation |  |  |  |
|  | - Counterbalance valve incorporated preventing unintentional boom dropping in the event of a hose failure |  |  |  |
| Load Sensing | - Pressure transducer sensing coupled with 3-axis true boom angle measurement relative to crane mounting surface and 3-axis true crane base attitude monitoring (truck level). |  |  |  |
|  | - Automatic 50% reduction of rotation speed while under load (slow rotate) |  |  |  |
|  | - In Overload condition, disable only hoist up, boom extend and boom down functions. Other functions shall remain operable |  |  |  |
|  | - In Low Boom Pressure condition, disable only boom down, swing CW and CCW, boom retract and extend, hoist up. Other functions remain operable (Anti-Bridging) |  |  |  |
| Extension | Hydraulically actuated by means of extension cylinder from 8 ft. to 12 ft. and manually actuated from 12 ft. to 16 ft. |  |  |  |
|  | Extension cylinder: - Trunnion mounted inside boom to protect from external damage |  |  |  |
|  | - Double acting for smooth operation |  |  |  |
|  | - Incorporates counterbalance valve to prevent unintentional boom retraction in the event of a hose failure |  |  |  |
|  | 4 ft. manual extension |  |  |  |
|  | Total 16 ft. extension |  |  |  |
| ###### Hoist | Hydraulically motor-driven planetary gear reduction with a load holding brake |  |  |  |
|  | Rated @ 2,000 lb. last layer capacity |  |  |  |
|  | Line speed of 20 ft./min. (single line) with no load |  |  |  |
|  | 80-ft. of .3125 in. diameter aircraft cable with 9,800 lb. breaking strength compliance to ANSI Standards |  |  |  |
| ###### Sheave | Anti-friction, high strength polymer with maintenance free, sealed needle bearings |  |  |  |
| Anti-two-block Preventer | A bail type to reduce crown to hook distance. (Hanging block two-block sensor is not acceptable.) |  |  |  |
|  | When activated, shall disable hoist up, boom down and extend out functions |  |  |  |
|  | Located on the left side of the boom to protect it from tree limbs, etc. while traveling |  |  |  |
|  | Cable shall be contained in a high impact plastic case with a spring loaded cable reel |  |  |  |
| Swivel Block | - Heavy-duty type with hook with latch |  |  |  |
|  | - Provision for double line string up |  |  |  |
| ###### Rotation | 370° rotation range |  |  |  |
|  | 40:1 worm gear set |  |  |  |
|  | - Provision for adjusting backlash between worm & gear - (Non-adjustable gear set is NOT acceptable.) |  |  |  |
|  | - Turntable style slew bearings |  |  |  |
|  | Cycle speed – 38 sec. no load |  |  |  |

| Hydraulic System | Main valve – Mono-block type with individual cartridge valves for the crane functions for easy accessibility and serviceability |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Proportional Boom Up/Down |  |  |  |
|  | - Proportional Hoist Up/Down |  |  |  |
|  | - Proportional Rotation |  |  |  |
|  | - Non-Proportional Boom Ext/Ret. |  |  |  |
|  | - Manual Override on all functions |  |  |  |

| Control System | Remote Control – FM Radio & Tethered Options: |  |  |  |
| --- | --- | --- | --- | --- |
|  | - Handheld remote control designed for one hand operation of crane functions. |  |  |  |
|  | - 2 joystick / 1 trigger configuration |  |  |  |
|  | - IP67 and CE certified remote |  |  |  |
|  | - 100% fully independent proportional function joystick or trigger control |  |  |  |
|  | - Integrated E-Stop button on remote |  |  |  |
|  | - Four distinct transmitter signal output settings to maximize control resolution for creep operation |  |  |  |
|  | - Transmitter shall have a color LCD readout providing the operator with feedback to allow explanation of current operating status |  |  |  |
|  | - Dock for remote charging and storage included |  |  |  |
|  | - Alkaline battery (AA x 3) backup included |  |  |  |
|  | Remote Control – FM Radio: |  |  |  |
|  | - Operates on a rechargeable, internal, lithium-ion battery |  |  |  |
|  | - Has an estimated 300 foot operating range |  |  |  |
|  | Remote Control – Tethered: |  |  |  |
|  | - 32 ft. communication cable included |  |  |  |
|  | Receiver – FM Radio and tethered Remote - Crane mounted and features - IP65 rated |  |  |  |
|  | See Options Section below for additional control configurations. |  |  |  |

| Warranty | Manufacturer shall provide a two-year limited warranty against defects in materials and workmanship from the date of delivery |  |  |  |
| --- | --- | --- | --- | --- |
| Options (Must be available as required) | Paint – provide code and desired color if other than standard |  |  |  |
|  | Boom Support shall be installed for the boom in a stowed position while in transit |  |  |  |
|  | Outriggers: - Shall be installed to increase stability and reduce the load on the truck springs while lifting |  |  |  |
|  | Oil Temperature Sensor to monitor crane hydraulic oil temperatures |  |  |  |
| Notes: |  |  |  |  |
| Submitted by: |  |  | Date: |  |

Figure 1: EHC-4, 16’ Boom Load Chart